# Two-Pass Scanning with Backtracking (Improved Detection Logic)

Input: Calibrated IR sensor readings
Output: Raw position, peak, and width data for both scans

Parameters:

- Vbase

- Vback

- Vscan2 ← speeds

- Tth ← detection threshold

- kth ← adaptive threshold factor

- texit, twait1, twait5 ← timing constants

- α ← EMA coefficient

### Initialisation

- Initialise motors, encoders, kinematics, PID, and sensors

- Initialise timers and variables

- Apply EMA filtering

### Calibration and Wait

- Rotate robot and calibrate sensors

- If elapsed time > twait5 → motors ← stop

- Wait for twait5

## Scan 1 (Forward Pass with Explicit Detection)

- scanArmed ← false

- lastDetectTime ← 0

- For each sensor i

- inTarget₁[i] ← false

- P1[i]←0

- End For

- motors ← drive forward with speed Vbase using PID

- While (scanArmed = false) OR (currentTime − lastDetectTime < texit)

- Update position x

- Read filtered sensor values vi

- For each sensor i

- detected ← (vi>Tth)

- If detected = true

- scanArmed ← true

- lastDetectTime ← currentTime

- If inTarget₁[i] = false // entering

- inTarget₁[i] ← true

- X1,start[i]←x

- End If

- P1[i]←max⁡(P1[i],vi)

- Else

- If inTarget₁[i] = true // leaving

- inTarget₁[i] ← false

- X1,end[i]←x

- End If

- End If

- End For

- End While

- motors ← stop

## Backtrack Phase

- Wait for twait1

- motors ← drive backward with speed Vback using PID

- While detection active or within short delay

- Read sensor values

- End While

- motors ← stop

- Wait for twait1

## Scan 2 (Adaptive Detection with Same Logic)

- scanArmed₂ ← false

- lastDetectTime₂ ← 0

- For each sensor i

- inTarget₂[i] ← false

- P2[i]←0

- Tth2[i]←kth×P1[i]

- End For

- motors ← drive forward with speed Vscan2 using PID

- While (scanArmed₂ = false) OR (currentTime − lastDetectTime₂ < texit)

- Update position x

- Read filtered sensor values vi

- For each sensor i

- detected ← (vi>Tth2[i])

- If detected = true

- scanArmed₂ ← true

- lastDetectTime₂ ← currentTime

- If inTarget₂[i] = false // entering

- inTarget₂[i] ← true

- X2,start[i]←x

- End If

- P2[i]←max⁡(P2[i],vi)

- Else

- If inTarget₂[i] = true // leaving

- inTarget₂[i] ← false

- X2,end[i]←x

- End If

- End If

- End For

- End While

- motors ← stop

## Two-Pass Scanning with Backtracking (Improved Detection Logic)

The system implements a structured two-pass scanning strategy to detect and measure target regions using IR sensors combined with encoder-based position tracking. The objective is to improve robustness against noise and environmental variation while achieving accurate estimates of target position, peak response, and spatial width.

### Overview of Operation

The algorithm is divided into four main stages:

- Initialisation and Calibration

- First Scan (Exploratory Detection Pass)

- Backtracking Repositioning

- Second Scan (Refined Adaptive Detection Pass)

Each stage contributes to improving measurement reliability, repeatability, and robustness.

## 1. Initialisation and Calibration

At startup, all subsystems are initialised, including motors, encoders, kinematics, PID control, and IR sensors. An exponential moving average (EMA) filter is applied to all sensor readings to reduce high-frequency noise and improve detection stability.

The EMA coefficient α is selected based on prior experimental tuning. It is chosen to balance noise suppression and responsiveness: a lower value improves smoothing but introduces lag, while a higher value increases responsiveness but allows more noise to pass through. The selected value represents a compromise that ensures stable threshold crossings while maintaining sufficient temporal responsiveness for edge detection.

The system uses an Exponential Moving Average (EMA) filter rather than alternative filtering methods such as boxcar (moving average) or Butterworth filters due to real-time performance and detection accuracy requirements.

The EMA filter is selected primarily because it provides a low-latency response, which is critical for real-time threshold detection and accurate edge localisation. Unlike a boxcar filter, which computes a uniform average over a fixed window, EMA applies exponentially decreasing weighting to older samples. This allows the system to react more quickly to sudden changes in sensor readings, which is essential when detecting sharp transitions at target boundaries.

In contrast, a boxcar filter introduces a noticeable phase delay proportional to its window size. While it effectively reduces noise, this delay shifts the apparent position of signal edges, leading to systematic errors in boundary detection and reduced accuracy in encoder-based width estimation. This makes it unsuitable for applications where precise timing of threshold crossings is required.

A Butterworth filter provides strong frequency-domain noise suppression and a smooth output signal; however, it requires higher computational overhead due to recursive filtering and typically introduces phase distortion unless implemented as a zero-phase offline filter. In a real-time embedded system, this added computational cost and latency are undesirable, particularly when maintaining high sampling rates for multiple sensors.

The EMA filter therefore provides the best balance between noise reduction and responsiveness. It is computationally lightweight, requiring only a single multiplication and addition per sample, which allows for high-frequency sensor updates without degrading system performance. This efficiency supports a higher effective sampling rate, which directly improves spatial resolution in encoder-based tracking.

Most importantly, EMA preserves the temporal integrity of signal transitions, enabling accurate real-time edge detection, which is essential for reliable start and end position recording in both scan passes.

A calibration phase is then performed to establish baseline sensor behaviour and noise characteristics. After calibration, the robot remains stationary for a fixed delay twait5 to allow sensor stabilisation before motion begins.

## 2. Scan 1: Exploratory Forward Detection Pass

The first scan acts as a broad detection and estimation stage, operating under a fixed threshold Tth1 and a constant forward velocity Vbase.

### Motion Parameter: Vbase

The forward scan velocity Vbase is chosen empirically from previous testing. It represents a trade-off between spatial resolution and scan duration. A moderate speed is required so that the robot does not traverse target regions too quickly, ensuring sufficient sampling density across sensor peaks. However, it must not be excessively slow, as this would increase scan time without significant improvement in measurement quality. Therefore, Vbase is selected as a balanced operating point that allows reliable peak observation while maintaining efficiency.

### Detection Threshold: Tth1

The initial threshold Tth1 is determined experimentally based on observed sensor noise levels during calibration trials. It is set above the noise floor to avoid false detections while remaining sensitive enough to detect weak target responses. This ensures robust initial segmentation of target regions during Scan 1.

### Detection Logic

During motion, filtered sensor values vi are continuously evaluated against Tth1. Each sensor maintains a state variable inTarget[i] to detect transitions:

- Entry event: vi>Tth1 and previously inactive
→ record start position X1,start[i]

- Exit event: vi≤Tth1 after being active
→ record end position X1,end[i]

The maximum observed response is stored as:

P1[i]=max⁡(P1[i],vi)

### Scan Control Logic

A scanArmed flag ensures that timing logic only activates after the first valid detection, preventing premature termination when starting on a non-target surface. Additionally, an exit timeout texit ensures that brief signal dropouts do not fragment a continuous target into multiple detections.

### Motion Speed Justification: Backtracking Context

Although Scan 1 uses forward motion, it is important to note that subsequent reverse motion (backtracking) is intentionally handled separately due to inherent asymmetry in robot dynamics. Reverse motion is typically less accurate due to increased wheel slip, motor nonlinearity, and reduced odometry reliability. However, this limitation is acceptable because it is only used for coarse repositioning rather than measurement.

## 3. Backtracking Repositioning Phase

After Scan 1, the robot performs a controlled reverse motion at velocity Vback.

### Motion Parameter: Vback

The backtracking speed is derived relative to Vbase but is typically set slightly slower. This is because reverse motion is mechanically less stable and more prone to odometry error in differential drive systems. A reduced speed improves control stability and reduces overshoot, ensuring that the robot returns to a consistent pre-scan region.

The purpose of this stage is not measurement accuracy but spatial reset, ensuring that Scan 2 begins from a controlled region. Therefore, high precision is not required.

A short stabilisation delay is included after backtracking before the next scan begins.

## 4. Scan 2: Adaptive Refinement Pass

The second scan repeats the same structural detection logic but introduces adaptive thresholding and increased measurement precision.

### Motion Parameter: Vscan2

The second scan velocity Vscan2 is intentionally set as the slowest motion speed. This is because Scan 2 is the refinement stage, where accuracy is prioritised over execution time. A slower velocity increases encoder sampling density, improving spatial resolution of:

- peak detection

- boundary estimation

- width measurement

It also increases the time spent over high-response regions, improving peak stability and reducing sensitivity to transient noise.

### Adaptive Threshold: Tth2

Each sensor uses an adaptive threshold defined as:

Tth2[i]=0.5⋅P1[i]

This is based on the assumption that when a target occupies approximately half of the sensor’s field of view, the observed signal approximates a representative midpoint of the full response peak. Using 50% of the previously observed peak allows the system to scale detection sensitivity relative to actual signal strength.

This approach provides several benefits:

- compensates for sensor-to-sensor variation

- adapts to different surface reflectivities

- reduces dependence on absolute threshold tuning

- improves robustness across varying environmental conditions

### Detection Logic (Scan 2)

Scan 2 uses the same edge detection mechanism as Scan 1, with inTarget[i] tracking entry and exit events. However, thresholding is now adaptive per sensor, improving consistency of boundary detection.

A second scanArmed₂ mechanism and exit timeout texit are again used to ensure robust segmentation and prevent noise-induced fragmentation.

The maximum response is recorded as:

P2[i]=max⁡(P2[i],vi)

## Key System Design Principles

### 1. Scan Arming Mechanism

The scanArmed condition ensures that exit timing only begins after a valid detection, preventing premature termination when starting on non-target regions.

### 2. Exit Delay Robustness

The texit parameter introduces temporal hysteresis, preventing short signal dropouts from incorrectly splitting a single continuous target into multiple detections.

### 3. Edge Detection via State Tracking

The inTarget[i] flag enables detection of transitions rather than relying solely on instantaneous threshold crossings. This ensures:

- one start event per entry

- one end event per exit

- stable width estimation based on encoder position

### 4. Two-Pass Strategy Justification

The system separates detection into:

- Scan 1: exploratory detection using fixed thresholds

- Scan 2: refined measurement using adaptive thresholds derived from Scan 1

This structure improves robustness against:

- sensor noise

- surface reflectivity variation

- threshold sensitivity issues

- environmental drift

### 5. Backtracking as Coarse Repositioning

The backtracking phase ensures repeatable scan conditions without requiring high positional accuracy. Since Scan 2 re-establishes measurement precision, reverse motion error does not significantly affect final results.